MAYIM TOOLS

User Manual for QGIS 4+

Version 0.1.0

Status: Development / Technical Preview

Prepared for Mayim Consulting

Plugin repository: https://github.com/chrismayim/mayim-tools

Licence: GNU General Public License v2 or later

# Document Control

## Document Information

| Item | Detail |
| --- | --- |
| Product | Mayim Tools |
| Platform | QGIS 4+ |
| Current tested version | QGIS 4.0.3 |
| Manual version | 0.1.0 |
| Plugin status | Active development |
| Organisation | Mayim Consulting |
| Repository | https://github.com/chrismayim/mayim-tools |
| Licence | GNU General Public License v2 or later |
| Primary domain | Hydrology and geospatial engineering |
| Audience | GIS users, engineers, hydrologists, analysts and reviewers |

## Document Status

This manual is a living document. It will be updated as new tools, categories, processing parameters, outputs, and known limitations are added to Mayim Tools.

The manual describes the current development version and should not be interpreted as a substitute for project-specific engineering judgement, data validation, regulatory requirements, or independent hydrological review.

## Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1.0 | 2026-08-22 | Initial draft manual covering Screening and Smoothing tools | Mayim Consulting |

# Contents

Document Control	2

Document Information	2

Document Status	2

Revision History	2

1. Introduction	3

1.1 Purpose of This Manual	3

1.2 What Is Mayim Tools?	3

1.3 Intended Use	4

1.4 Important Distinction: Screening, Artifact Correction and Conditioning	4

Artifact correction	4

Hydrological enforcement	4

2. About Mayim Tools	6

2.1 Accessing the Plugin	6

2.2 Categories	6

Hydrology Tools	6

Geometry Tools	6

2.3 Processing Toolbox Integration	6

3. Installation and Availability	8

3.1 Development Installation	8

3.2 Public Distribution	8

3.3 Requirements	8

4. Getting Started	9

4.1 Open QGIS	9

4.2 Load Source Data	9

4.3 Open the Processing Toolbox	9

4.4 Read the Tool Help Before Running	9

4.5 Review All Outputs	9

5. General Processing Toolbox Concepts	11

5.1 Input Layers	11

5.2 Temporary Outputs	11

5.3 Output Folder	11

5.4 Load-Output Options	11

6. Data and Coordinate Reference System Requirements	12

6.1 DEM Source	12

6.2 Coordinate Reference System	12

6.3 Resolution	12

6.4 Vertical Accuracy	13

7. Outputs, Reports and Provenance	14

7.1 Raster Outputs	14

7.2 Text Reports	14

7.3 Provenance JSON	14

7.4 Difference and Modification Outputs	15

8. Hydrology Tools	16

8.1 DEM Hydrological Screening	16

Purpose	16

Processing ID	16

Main Functions	16

Important Limitation	17

Recommended Workflow	17

Outputs	17

Interpretation	17

8.2 DEM Hydrological Smoothing	18

Purpose	18

Processing ID	18

Method	18

When to Use It	18

Recommended Starting Parameters	18

Parameter Guidance	18

Outputs	19

Interpretation	19

8.3 Future Tool	19

8.4 Future Tool	19

9. Recommended Hydrological Workflow	20

Stage 0 – Ingestion and QA	20

Stage 1 – Artifact Screening	20

Stage 2 – Controlled Smoothing	20

Stage 3 and Beyond – Planned	20

10. Quality Assurance and Interpretation	21

11. Troubleshooting	22

12. Versioning and Change Management	23

13. References	24

Appendix A — Processing Parameter Conventions	25

Appendix B — Output Naming Conventions	26

Appendix C — Glossary	27

# 1. Introduction

## 1.1 Purpose of This Manual

This manual explains how to install, configure, and use Mayim Tools in QGIS 4+. It provides practical guidance for running the available Processing Toolbox algorithms, interpreting their outputs, and applying the results in a reproducible hydrological workflow.

The manual is structured so that each Mayim Tools algorithm has its own chapter. Each tool chapter explains:

- The purpose of the tool.

- Where it fits into the hydrological workflow.

- Required and optional inputs.

- Recommended parameter values.

- Processing assumptions.

- Outputs produced.

- Quality-control checks.

- Limitations and known issues.

- Example usage.

## 1.2 What Is Mayim Tools?

Mayim Tools is a publicly redistributable QGIS plugin for engineering and geospatial analysis. It provides a collection of modular tools organised into functional categories.

The initial development focus is hydrological analysis, particularly the preparation of digital elevation models for subsequent terrain and flow-routing operations.

Mayim Tools is designed to be:

- Modular.

- Transparent.

- Reproducible.

- Resolution-aware.

- Suitable for international DEM sources.

- Integrated with the QGIS Processing Framework.

- Supported by reports and provenance records.

- Extensible with additional tools and categories.

## 1.3 Intended Use

Mayim Tools is intended to support workflows such as:

- Digital elevation model quality assessment.

- DEM artifact screening.

- Controlled DEM smoothing.

- Depression analysis.

- Hydrological conditioning.

- Flow-direction preparation.

- Flow-accumulation preparation.

- Stream-network extraction.

- Engineering and environmental GIS analysis.

The plugin does not replace professional judgement. Its outputs must be reviewed in relation to:

- The source data.

- Survey accuracy.

- Terrain characteristics.

- Land cover.

- Known structures.

- Hydrological observations.

- Project requirements.

- Applicable standards and regulations.

## 1.4 Important Distinction: Screening, Artifact Correction and Conditioning

Mayim Tools follows the distinction between:

### Artifact correction

Improving the DEM so that it better represents the underlying terrain.

### Hydrological enforcement

Modifying the terrain so that flow-routing algorithms can operate consistently.

These are not the same operation. For example, a DEM may contain:

- Vegetation or building bias.

- Radar or InSAR noise.

- NoData voids.

- Bridges or structures that interrupt apparent drainage.

- Genuine closed basins.

- Interpolation artifacts.

A robust workflow should not automatically erase every depression. It should first assess the data and distinguish possible artifacts from potentially meaningful terrain.

# 2. About Mayim Tools

## 2.1 Accessing the Plugin

Mayim Tools integrates with QGIS through the Processing Framework. Tools are intended to be available through:

Processing
└── Toolbox
    └── Mayim Tools
        └── Hydrology Tools

The plugin also provides a Mayim Tools dock panel for browsing categories and available tools.

## 2.2 Categories

### Hydrology Tools

Tools for:

- DEM screening.

- DEM smoothing.

- Depression processing.

- Flow-routing preparation.

- Future hydrological analysis functions.

### Geometry Tools

A framework category for future geometric and spatial-processing tools.

## 2.3 Processing Toolbox Integration

Processing Toolbox algorithms can generally be used in:

- The Processing Toolbox.

- Batch Processing.

- The QGIS Graphical Modeler.

- QGIS processing history.

- The QGIS Python Console.

- Automated or repeatable workflows.

A Processing algorithm can typically be called from Python using a command such as:

processing.run(
    "mayimtools:demhydrologicalsmoothing",
    {
        # Parameters supplied here
    },
)

The exact parameter names and values should always be confirmed from the relevant tool chapter or QGIS Processing dialog.

# 3. Installation and Availability

## 3.1 Development Installation

During development, Mayim Tools is deployed locally into the QGIS user plugin directory.

The development deployment location on the current reference workstation is:

C:\Users\caets\AppData\Roaming\QGIS\QGIS4\profiles\default\python\plugins\mayim_tools

This is a developer installation and is not the same as public distribution through the QGIS Plugin Repository.

## 3.2 Public Distribution

A public release may eventually be distributed through:

- The official QGIS Plugin Repository.

- GitHub releases.

- A project-specific organisational distribution channel.

A development or experimental installation should not be used in production without appropriate testing and review.

## 3.3 Requirements

The current development environment is:

| Component | Version |
| --- | --- |
| QGIS | 4.0.3 |
| Python | 3.12.13 |
| Qt | 6.11.1 |
| GDAL | 3.13.0 |
| PyQGIS | QGIS 4 API |
| Operating system | Windows |
| Raster processing | rasterio, NumPy, SciPy |

Tool-specific dependencies should be documented in the relevant tool chapter.

# 4. Getting Started

## 4.1 Open QGIS

Start QGIS 4 and open or create a project.

## 4.2 Load Source Data

Load the DEM or other required source layers into the QGIS project using:

Layer
└── Add Layer
    └── Add Raster Layer

## 4.3 Open the Processing Toolbox

Open:

Processing
└── Toolbox

Search for:

Mayim Tools

Expand the relevant category and double-click the required algorithm.

## 4.4 Read the Tool Help Before Running

Before execution, review:

- The tool description.

- The recommended parameter values.

- The valid parameter ranges.

- The output definitions.

- Any warnings or limitations.

## 4.5 Review All Outputs

Do not rely only on the primary raster output. Where available, also review:

- Difference rasters.

- Masks.

- Reports.

- Provenance files.

- Warnings.

- Processing feedback.

# 5. General Processing Toolbox Concepts

## 5.1 Input Layers

Input layers are normally selected from the layers currently loaded in QGIS. Some tools may also accept file paths or external datasets.

## 5.2 Temporary Outputs

When TEMPORARY_OUTPUT or an equivalent temporary destination is selected, QGIS writes the result into a temporary processing directory.

Temporary outputs may be removed when the QGIS session ends or when temporary files are cleaned. Important results should therefore be written to a permanent project or data directory.

## 5.3 Output Folder

Where a tool provides an output-folder parameter, use a permanent folder when results are required for:

- Project records.

- Peer review.

- Reproducibility.

- Regulatory submissions.

- Future processing stages.

Avoid storing final project outputs only in a temporary directory.

## 5.4 Load-Output Options

Where available, load checkboxes allow users to select which generated layers should be added to the QGIS project.

Loading a layer does not necessarily mean that the file has been permanently archived. The output file should still be stored in an appropriate permanent location.

# 6. Data and Coordinate Reference System Requirements

## 6.1 DEM Source

Mayim Tools is intended to support valid DEM sources from different providers and resolutions, including:

- LiDAR-derived terrain models.

- LiDAR-derived surface models.

- SRTM.

- Copernicus GLO-30.

- FABDEM and similar corrected global products.

- Aerial photogrammetry.

- Other valid raster elevation products.

The user should identify the source type whenever this information is known.

## 6.2 Coordinate Reference System

The input DEM should have an assigned CRS.

If the CRS is missing:

- The tool may be able to perform local raster operations.

- Distance-dependent or area-dependent calculations may not be reliable.

- The user should assign or define the correct CRS before further hydrological processing.

If the DEM uses a geographic CRS, latitude/longitude units are in degrees. This may be acceptable for some local, cell-based operations, but it is unsuitable for many calculations involving:

- Distances.

- Areas.

- Breach lengths.

- Cell-size measurements in metres.

- Resolution-based thresholds.

Where metric calculations are required, reproject the DEM to a suitable projected CRS and review the resampling method.

## 6.3 Resolution

The native raster resolution should be retained and documented. Mayim Tools uses resolution-aware parameters in selected tools, but resolution adaptation does not remove the need for professional review.

A 1 m LiDAR DEM and a 30 m global DEM should not automatically be interpreted as equivalent surfaces.

## 6.4 Vertical Accuracy

Where available, provide a formal vertical accuracy value such as:

- RMSE.

- LE90.

- A source-specific accuracy estimate.

- A survey-derived accuracy assessment.

If no formal value is available, the tool may use a conservative estimate based on the selected source type. The estimate must be treated as metadata guidance, not as a measured accuracy result.

# 7. Outputs, Reports and Provenance

## 7.1 Raster Outputs

Mayim Tools may produce:

- Processed DEMs.

- Classification masks.

- Artifact masks.

- Difference rasters.

- Smoothing masks.

- Diagnostic rasters.

Each raster output should be reviewed for:

- CRS.

- Extent.

- Resolution.

- NoData handling.

- Data type.

- Unexpected values.

- Alignment with the source DEM.

## 7.2 Text Reports

Text reports provide a human-readable summary of:

- Input data.

- Parameters.

- Warnings.

- Processing results.

- Output file locations.

- Recommended next steps.

- Interpretation guidance.

Text reports should be archived with the corresponding output rasters.

## 7.3 Provenance JSON

The provenance JSON is intended for machine-readable auditability. It may contain:

- Tool name.

- Algorithm ID.

- Tool version.

- Timestamp.

- Input path.

- Processing parameters.

- Output paths.

- Statistics.

- Warnings.

- Processing decisions.

The provenance file should be retained whenever the result is used in a formal engineering or analytical workflow.

## 7.4 Difference and Modification Outputs

A difference raster typically shows:

processed value - original value

Interpretation depends on the tool. For a smoothing operation:

- Negative values indicate a decrease.

- Positive values indicate an increase.

- Zero indicates no effective change.

Modification masks identify cells changed by the operation and support visual inspection and review.

# 8. Hydrology Tools

This chapter documents each Hydrology Tools algorithm currently available in Mayim Tools. Each tool follows the repeatable chapter structure described in Appendix A, covering purpose, pipeline stage, inputs, parameters, outputs, interpretation, and known limitations.

## 8.1 DEM Hydrological Screening

### Purpose

DEM Hydrological Screening implements the initial DEM review stages:

- Stage 0: Ingestion and QA.

- Stage 1: Artifact screening.

It is intended to be run before later smoothing, depression processing, flow-direction calculation, or flow-accumulation analysis.

### Processing ID

mayimtools:demhydrologicalscreening

### Main Functions

The tool:

- Reads DEM metadata.

- Checks the assigned CRS.

- Reports raster dimensions.

- Reports native resolution.

- Reports NoData information.

- Assigns or estimates vertical accuracy.

- Detects NoData regions.

- Classifies void regions by size.

- Interpolates small voids where possible.

- Reports medium and large voids.

- Applies a local MAD-based artifact screening process.

- Produces screening masks.

- Produces a screened DEM.

- Produces a text report.

- Produces a JSON provenance record.

### Important Limitation

The current tool performs artifact screening, but it does not constitute a complete implementation of every possible Stage 1 correction method.

In particular, the following may remain separate or future functions:

- Full LiDAR ground filtering.

- SMRF implementation.

- Progressive TIN densification.

- Machine-learning bias correction.

- Bare-earth substitution.

- Co-registered land-cover correction.

### Recommended Workflow

- Load the DEM into QGIS.

- Confirm that the DEM source type is known.

- Run DEM Hydrological Screening.

- Review the CRS warning or confirmation.

- Review the void mask.

- Review the artifact mask.

- Read the text report.

- Archive the provenance JSON.

- Decide whether smoothing or further artifact correction is justified.

- Do not proceed blindly to depression processing.

### Outputs

Typical outputs include:

<dem>_screened.tif
<dem>_void_mask.tif
<dem>_artifact_mask.tif
<dem>_screening_report.txt
<dem>_provenance.json

### Interpretation

A low artifact percentage does not prove that the DEM is correct. It indicates that relatively few cells were flagged by the selected MAD parameters.

A large void region should not be silently treated as valid terrain. It should be investigated as a possible data-collection gap.

A geographic CRS warning should be considered before operations requiring metric distances or areas.

## 8.2 DEM Hydrological Smoothing

### Purpose

DEM Hydrological Smoothing implements Stage 2: Controlled Smoothing.

It is intended to reduce residual high-frequency noise after Stage 1 screening, while preserving stronger terrain breaks.

### Processing ID

mayimtools:demhydrologicalsmoothing

### Method

The current implementation uses Perona-Malik-style anisotropic diffusion.

This is an edge-preserving approach that reduces diffusion across stronger elevation gradients. It is therefore preferable to applying an indiscriminate uniform low-pass filter where preservation of terrain breaks is important.

### When to Use It

Use this tool only when:

- Stage 1 screening indicates residual high-frequency noise.

- The artifact mask and report have been reviewed.

- Smoothing is justified by the DEM characteristics.

- The analyst is prepared to inspect the difference and modification outputs.

Do not use smoothing simply because the DEM contains natural terrain variation.

### Recommended Starting Parameters

| Parameter | Recommended starting value | Valid range |
| --- | --- | --- |
| Diffusion iterations | 5 | 1–100 |
| Diffusion strength | 0.20 | 0.01–0.25 |
| Edge threshold | 1.0 elevation unit | Greater than 0 |
| Resolution scaling | Enabled | True / False |

### Parameter Guidance

Diffusion iterations

Start with 5 iterations. Use approximately 3–5 iterations for light noise. Consider 5–10 only where persistent high-frequency noise remains.

Increasing iterations increases the cumulative amount of smoothing and may alter genuine terrain features.

Diffusion strength

Start with 0.20. The current implementation limits this parameter to 0.25 for stability.

Higher values produce stronger updates per iteration, but stronger is not necessarily better.

Edge threshold

The edge threshold is expressed in the elevation units of the input DEM.

A lower value preserves terrain edges more strongly and reduces smoothing across elevation breaks. A higher value allows more smoothing across larger gradients and may reduce the distinction between noise and genuine terrain.

Where a reliable RMSE is available, an initial sensitivity test around one to two times the RMSE may be useful. This is only a starting heuristic and is not an automatic accuracy correction.

### Outputs

Typical outputs include:

<dem>_smoothed.tif
<dem>_smoothing_difference.tif
<dem>_smoothing_mask.tif
<dem>_smoothing_report.txt
<dem>_smoothing_provenance.json

### Interpretation

Review the smoothed DEM, the difference raster, and the modification mask together. Confirm that changes are concentrated in areas of expected high-frequency noise and that genuine terrain breaks (e.g. banks, channels, structures) have been substantially preserved.

A smoothing result should not be accepted purely on the basis that it looks visually cleaner. The difference raster should be checked against the artifact mask produced during Stage 1 screening to confirm the changes are consistent with the flagged noise.

## 8.3 Future Tool

Reserved for a future Hydrology Tools algorithm. This chapter will follow the same structure as Sections 8.1 and 8.2 once the corresponding tool is implemented and documented.

## 8.4 Future Tool

Reserved for a future Hydrology Tools algorithm.

# 9. Recommended Hydrological Workflow

The following sequence reflects the current staged pipeline supported by Mayim Tools. Later stages (depression processing, flow-direction and flow-accumulation preparation, stream-network extraction) are anticipated but not yet implemented as of manual version 0.1.0.

### Stage 0 – Ingestion and QA

Load the source DEM, confirm the CRS, and record metadata (resolution, NoData convention, source type, and any known vertical accuracy value).

### Stage 1 – Artifact Screening

Run DEM Hydrological Screening. Review the void mask, artifact mask, and text report before deciding whether further correction is warranted.

### Stage 2 – Controlled Smoothing

Where residual high-frequency noise remains after screening, run DEM Hydrological Smoothing using the recommended starting parameters. Review the difference raster and modification mask.

### Stage 3 and Beyond – Planned

Depression analysis, hydrological conditioning, flow-direction preparation, flow-accumulation preparation, and stream-network extraction are planned as future Hydrology Tools chapters and will be documented here as they become available.

# 10. Quality Assurance and Interpretation

Every Mayim Tools output should be treated as an input to professional review, not as a final result in its own right. As a general QA discipline:

- Confirm the CRS and resolution of every output against the source DEM.

- Cross-check masks (void, artifact, modification) against the corresponding text report.

- Retain the JSON provenance record for any output used in a formal deliverable.

- Compare difference rasters against known site features (structures, channels, vegetation) before accepting a correction.

- Avoid applying successive corrections (screening, smoothing, or future conditioning tools) without reviewing the outputs of each preceding stage.

- Document the rationale for any parameter values that differ from the recommended starting values.

# 11. Troubleshooting

| Symptom | Likely cause | Suggested action |
| --- | --- | --- |
| Tool does not appear in the Processing Toolbox | Plugin not installed in the active QGIS profile, or not enabled | Confirm the plugin folder is in the correct profile's plugins directory and enable it via Plugin Manager |
| CRS warning on load | DEM has no assigned CRS or uses a geographic CRS | Assign or reproject the CRS before running metric-dependent tools |
| Unexpectedly large void or artifact areas | Genuine data-collection gaps, or overly sensitive screening parameters | Inspect the source DEM directly and consider adjusting MAD-based screening thresholds |
| Smoothing removes real terrain features | Edge threshold set too high, or too many diffusion iterations applied | Reduce the edge threshold or iteration count and re-run, then re-check the difference raster |
| Outputs missing after a QGIS session ends | Output destination was set to a temporary location | Re-run the tool with a permanent output folder |

# 12. Versioning and Change Management

Mayim Tools and this manual are versioned independently but released together where practical. The current status of both is Development / Technical Preview.

- Manual changes are recorded in the Revision History table in the Document Control section.

- New tools are added as new numbered subsections under Section 8, following the standard chapter template in Appendix A.

- Breaking changes to a tool's parameters or Processing ID should be flagged clearly at the top of the relevant tool chapter.

- Users should confirm the manual version matches the installed plugin version before relying on specific parameter guidance.

# 13. References

- QGIS Processing Framework documentation.

- PyQGIS API documentation (QGIS 4 API).

- Perera, P. and Malik, J., "Scale-space and edge detection using anisotropic diffusion," as the methodological basis for the Stage 2 smoothing implementation.

- GDAL raster data model documentation.

- Relevant national and regional hydrological design guidance applicable to the project context (e.g. drainage manuals and design standards used by the reviewing authority).

# Appendix A — Processing Parameter Conventions

Where practical, Mayim Tools parameters follow these conventions:

- Distances and lengths are expressed in the linear unit of the input CRS (typically metres for a projected CRS).

- Elevation-based thresholds are expressed in the vertical unit of the input DEM.

- Boolean parameters default to the safer or more conservative setting where a default is required.

- Iteration counts and strength parameters include a stated valid range and a recommended starting value.

Repeatable Chapter Structure

Each future tool chapter under Section 8 (Hydrology Tools) will use the following structure:

- 1. Purpose

- 2. Pipeline stage

- 3. When to use the tool

- 4. Processing method

- 5. Required inputs

- 6. Optional inputs

- 7. Recommended parameter values

- 8. Processing steps

- 9. Outputs

- 10. Output interpretation

- 11. Quality-control checks

- 12. Warnings and limitations

- 13. Example workflow

- 14. Provenance and reporting

- 15. Related tools and next steps

This will keep the manual consistent as more tools are added.

# Appendix B — Output Naming Conventions

Output files generally follow the pattern <dem>_<suffix>.<ext>, where <dem> is the base name of the input DEM. Current suffixes in use:

| Suffix | Produced by | Description |
| --- | --- | --- |
| _screened.tif | DEM Hydrological Screening | Screened DEM raster |
| _void_mask.tif | DEM Hydrological Screening | NoData void classification mask |
| _artifact_mask.tif | DEM Hydrological Screening | MAD-based artifact mask |
| _screening_report.txt | DEM Hydrological Screening | Human-readable screening report |
| _provenance.json | DEM Hydrological Screening | Machine-readable provenance record |
| _smoothed.tif | DEM Hydrological Smoothing | Smoothed DEM raster |
| _smoothing_difference.tif | DEM Hydrological Smoothing | Processed minus original elevation difference |
| _smoothing_mask.tif | DEM Hydrological Smoothing | Modified-cell mask |
| _smoothing_report.txt | DEM Hydrological Smoothing | Human-readable smoothing report |
| _smoothing_provenance.json | DEM Hydrological Smoothing | Machine-readable provenance record |

# Appendix C — Glossary

| Term | Definition |
| --- | --- |
| Artifact correction | Improving a DEM so that it better represents the underlying terrain. |
| Hydrological enforcement | Modifying terrain so that flow-routing algorithms can operate consistently. |
| MAD | Median Absolute Deviation; a robust statistic used for local artifact screening. |
| NoData void | A region of a raster with no valid elevation value. |
| Provenance record | A machine-readable log of the inputs, parameters, and decisions applied during a processing run. |
| Anisotropic diffusion | An edge-preserving smoothing technique (Perona-Malik) that limits diffusion across strong gradients. |
| Difference raster | A raster representing processed value minus original value, used to visualise the effect of a tool. |
| RMSE | Root Mean Square Error; a common measure of vertical accuracy for elevation data. |
| LE90 | Linear Error at the 90th percentile; a vertical accuracy measure. |
| CRS | Coordinate Reference System. |