MAYIM TOOLS

User Manual for QGIS 4+

Plugin version: 0.2.0
Manual version: 0.2.0

Status: Development / Technical Preview

Prepared for:
Mayim Consulting

Plugin repository:
https://github.com/chrismayim/mayim-tools

Licence:
GNU General Public License v2 or later

# Document Control

## Document Information

| Item | Detail |
| --- | --- |
| Product | Mayim Tools |
| Platform | QGIS 4+ |
| Tested version | QGIS 4.0.3 |
| Plugin version | 0.2.0 |
| Manual version | 0.2.0 |
| Plugin status | Active development / Technical Preview |
| Organisation | Mayim Consulting |
| Repository | https://github.com/chrismayim/mayim-tools |
| Licence | GNU General Public License v2 or later |
| Primary domain | Hydrology and geospatial engineering |
| Audience | GIS users, engineers, hydrologists, analysts and reviewers |

## Document Status

This manual is a living document and will be updated as new tools, categories, processing parameters, outputs, validation results and known limitations are added to Mayim Tools.

Version 0.2.0 documents the current development implementation of the Mayim Tools hydrological pipeline through Stage 6. The Stage 3–6 tools are technical-preview implementations and require appropriate data, parameter and engineering review before their outputs are used in formal deliverables.

This manual does not replace project-specific engineering judgement, data validation, regulatory requirements, survey information, hydrological observations or independent technical review.

## Revision History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| 0.1.0 | 2026-08-22 | Initial manual covering the Screening and Smoothing tools | Mayim Consulting |
| 0.2.0 | 2026-08-31 | Added Stage 3–6 tool chapters, updated workflow, outputs, provenance guidance and technical-preview limitations | Mayim Consulting |

# Contents

Document Control	2

Document Information	2

Document Status	2

Revision History	2

1. Introduction	3

1.1 Purpose of This Manual	3

1.2 What Is Mayim Tools?	3

1.3 Intended Use	4

1.4 Important Distinction: Screening, Artifact Correction and Conditioning	4

Artifact correction	4

Hydrological enforcement	4

2. About Mayim Tools	6

2.1 Accessing the Plugin	6

2.2 Categories	6

Hydrology Tools	6

Geometry Tools	6

2.3 Processing Toolbox Integration	6

3. Installation and Availability	8

3.1 Development Installation	8

3.2 Public Distribution	8

3.3 Requirements	8

4. Getting Started	9

4.1 Open QGIS	9

4.2 Load Source Data	9

4.3 Open the Processing Toolbox	9

4.4 Read the Tool Help Before Running	9

4.5 Review All Outputs	9

5. General Processing Toolbox Concepts	11

5.1 Input Layers	11

5.2 Temporary Outputs	11

5.3 Output Folder	11

5.4 Load-Output Options	11

6. Data and Coordinate Reference System Requirements	12

6.1 DEM Source	12

6.2 Coordinate Reference System	12

6.3 Resolution	12

6.4 Vertical Accuracy	13

7. Outputs, Reports and Provenance	14

7.1 Raster Outputs	14

7.2 Text Reports	14

7.3 Provenance JSON	14

7.4 Difference and Modification Outputs	15

8. Hydrology Tools	16

8.1 DEM Hydrological Screening	16

Purpose	16

Processing ID	16

Main Functions	16

Important Limitation	17

Recommended Workflow	17

Outputs	17

Interpretation	17

8.2 DEM Hydrological Smoothing	18

Purpose	18

Processing ID	18

Method	18

When to Use It	18

Recommended Starting Parameters	18

Parameter Guidance	18

Outputs	19

Interpretation	19

8.3 Future Tool	19

8.4 Future Tool	19

9. Recommended Hydrological Workflow	20

Stage 0 – Ingestion and QA	20

Stage 1 – Artifact Screening	20

Stage 2 – Controlled Smoothing	20

Stage 3 and Beyond – Planned	20

10. Quality Assurance and Interpretation	21

11. Troubleshooting	22

12. Versioning and Change Management	23

13. References	24

Appendix A — Processing Parameter Conventions	25

Appendix B — Output Naming Conventions	26

Appendix C — Glossary	27

# 1. Introduction

## 1.1 Purpose of This Manual

This manual explains how to install, configure, and use Mayim Tools in QGIS 4+. It provides practical guidance for running the available Processing Toolbox algorithms, interpreting their outputs, and applying the results in a reproducible hydrological workflow.

The manual is structured so that each Mayim Tools algorithm has its own chapter. Each tool chapter explains:

- The purpose of the tool.

- Where it fits into the hydrological workflow.

- Required and optional inputs.

- Recommended parameter values.

- Processing assumptions.

- Outputs produced.

- Quality-control checks.

- Limitations and known issues.

- Example usage.

## 1.2 What Is Mayim Tools?

Mayim Tools is a publicly redistributable QGIS plugin for engineering and geospatial analysis. It provides a collection of modular tools organised into functional categories.

The initial development focus is hydrological analysis, particularly the preparation of digital elevation models for subsequent terrain and flow-routing operations.

Mayim Tools is designed to be:

- Modular.

- Transparent.

- Reproducible.

- Resolution-aware.

- Suitable for international DEM sources.

- Integrated with the QGIS Processing Framework.

- Supported by reports and provenance records.

- Extensible with additional tools and categories.

## 1.3 Intended Use

Mayim Tools is intended to support workflows such as:

- Digital elevation model quality assessment.

- DEM artifact screening.

- Controlled DEM smoothing.

- Depression analysis.

- Hydrological conditioning.

- Flow-direction preparation.

- Flow-accumulation preparation.

- Stream-network extraction.

- Engineering and environmental GIS analysis.

The plugin does not replace professional judgement. Its outputs must be reviewed in relation to:

- The source data.

- Survey accuracy.

- Terrain characteristics.

- Land cover.

- Known structures.

- Hydrological observations.

- Project requirements.

- Applicable standards and regulations.

## 1.4 Important Distinction: Screening, Artifact Correction and Conditioning

Mayim Tools follows the distinction between:

### Artifact correction

Improving the DEM so that it better represents the underlying terrain.

### Hydrological enforcement

Modifying the terrain so that flow-routing algorithms can operate consistently.

These are not the same operation. For example, a DEM may contain:

- Vegetation or building bias.

- Radar or InSAR noise.

- NoData voids.

- Bridges or structures that interrupt apparent drainage.

- Genuine closed basins.

- Interpolation artifacts.

A robust workflow should not automatically erase every depression. It should first assess the data and distinguish possible artifacts from potentially meaningful terrain.

# 2. About Mayim Tools

## 2.1 Accessing the Plugin

Mayim Tools integrates with QGIS through the Processing Framework.

The Processing Toolbox is the primary interface:

Processing
└── Toolbox
    └── Mayim Tools
        └── Hydrology Tools
            ├── DEM Hydrological Screening
            ├── DEM Hydrological Smoothing
            ├── DEM Depression Analysis
            ├── DEM Hydrological Filling
            └── DEM Gradient Resolution

Mayim Tools also provides a dock panel for browsing categories and tools.

The main Mayim Tools plugin menu and toolbar are not currently used. Users should access tools through:

- Processing Toolbox.
- Mayim Tools dock panel.
- QGIS Processing history.
- QGIS Graphical Modeler.
- QGIS Python Console.

## 2.2 Categories

### Hydrology Tools

The Hydrology Tools category currently contains:

- DEM Hydrological Screening.

- DEM Hydrological Smoothing.

- DEM Depression Analysis.

- DEM Hydrological Filling.

- DEM Gradient Resolution.

These tools form a staged DEM analysis and conditioning workflow.

### Geometry Tools

Geometry Tools is currently a framework category reserved for future geometric and spatial-processing tools.

## 2.3 Processing Toolbox Integration

Processing Toolbox algorithms can generally be used in:

- The Processing Toolbox.

- Batch Processing.

- The QGIS Graphical Modeler.

- QGIS processing history.

- The QGIS Python Console.

- Automated or repeatable workflows.

A Processing algorithm can typically be called from Python using a command such as:

processing.run(
    "mayimtools:demhydrologicalsmoothing",
    {
        # Parameters supplied here
    },
)

The exact parameter names and values should always be confirmed from the relevant tool chapter or QGIS Processing dialog.

The Mayim Tools algorithms are designed to be used individually or as part of a staged workflow.

Examples of supported use include:

- Interactive execution from the Processing Toolbox.

- Batch Processing.

- QGIS Graphical Modeler.

- QGIS Processing history.

- QGIS Python Console.

- Reproducible processing sequences using saved parameters.

- Review of reports, masks and provenance records after each stage.

Example Python invocation:

processing.run(
    "mayimtools:demdepressionanalysis",
    {
        "INPUT_DEM": input_dem,
        "VERTICAL_ACCURACY": 0.15,
        "OUTPUT_FOLDER": output_folder,
    },
)

The exact parameter names and values should always be confirmed from the active QGIS Processing dialog because parameters may change between technical-preview versions.

# 3. Installation and Availability

## 3.1 Development Installation

During development, Mayim Tools is deployed locally into the QGIS user plugin directory.

The development deployment location on the current reference workstation is:

C:\Users\caets\AppData\Roaming\QGIS\QGIS4\profiles\default\python\plugins\mayim_tools

This is a developer installation and is not the same as public distribution through the QGIS Plugin Repository.

## 3.2 Public Distribution

A public release may eventually be distributed through:

- The official QGIS Plugin Repository.

- GitHub releases.

- A project-specific organisational distribution channel.

A development or experimental installation should not be used in production without appropriate testing and review.

## 3.3 Requirements

The current development environment is:

| Component | Version |
| --- | --- |
| QGIS | 4.0.3 |
| Python | 3.12.13 |
| Qt | 6.11.1 |
| GDAL | 3.13.0 |
| GEOS | 3.14.1 |
| PROJ | 9.8.1 |
| rasterio | 1.5.1 |
| NumPy | 2.4.6 |
| SciPy | 1.17.1 |

The current tools use:

- QGIS Processing API.
- PyQGIS.
- rasterio for raster I/O.
- NumPy for numerical array operations.
- SciPy for selected analysis operations.
- Python standard-library components.

The Stage 3–6 native components do not require RichDEM or WhiteboxTools at runtime.

The plugin is currently developed and tested on Windows. Additional operating-system validation is required before making broader compatibility claims.

# 4. Getting Started

## 4.1 Open QGIS

Start QGIS 4 and open or create a project.

## 4.2 Load Source Data

Load the DEM or other required source layers into the QGIS project using:

Layer
└── Add Layer
    └── Add Raster Layer

## 4.3 Open the Processing Toolbox

Open:

Processing
└── Toolbox

Search for:

Mayim Tools

Expand the relevant category and double-click the required algorithm.

## 4.4 Read the Tool Help Before Running

Before execution, review:

- The tool description.

- The recommended parameter values.

- The valid parameter ranges.

- The output definitions.

- Any warnings or limitations.

## 4.5 Review All Outputs

Do not rely only on the primary raster output. Where available, also review:

- Difference rasters.

- Masks.

- Reports.

- Provenance files.

- Warnings.

- Processing feedback.

# 5. General Processing Toolbox Concepts

## 5.1 Input Layers

Input layers are normally selected from the layers currently loaded in QGIS. Some tools may also accept file paths or external datasets.

## 5.2 Temporary Outputs

When TEMPORARY_OUTPUT or an equivalent temporary destination is selected, QGIS writes the result into a temporary processing directory.

Temporary outputs may be removed when the QGIS session ends or when temporary files are cleaned. Important results should therefore be written to a permanent project or data directory.

## 5.3 Output Folder

Where a tool provides an output-folder parameter, use a permanent folder when results are required for:

- Project records.

- Peer review.

- Reproducibility.

- Regulatory submissions.

- Future processing stages.

Avoid storing final project outputs only in a temporary directory.

## 5.4 Load-Output Options

Several Mayim Tools provide independent Boolean options controlling which generated raster outputs are loaded into the QGIS project.

Examples include:

Load screened DEM
Load void mask
Load artifact mask
Load smoothed DEM
Load smoothing difference
Load smoothing mask
Load depression ID raster
Load depression classification raster
Load preserved-topology DEM
Load hydrology-ready DEM
Load enforcement decision raster
Load gradient-resolved DEM
Load flat mask
Load flat-region IDs
Load gradient difference raster

Selected layers are loaded directly into the project and are not automatically placed in a Mayim Tools group.

Loading an output into QGIS does not guarantee that it has been permanently archived. Important outputs must be written to a permanent project-controlled folder.

# 6. Data and Coordinate Reference System Requirements

## 6.1 DEM Source

Mayim Tools is intended to support valid DEM sources from different providers and resolutions, including:

- LiDAR-derived terrain models.

- LiDAR-derived surface models.

- SRTM.

- Copernicus GLO-30.

- FABDEM and similar corrected global products.

- Aerial photogrammetry.

- Other valid raster elevation products.

The user should identify the source type whenever this information is known.

## 6.2 Coordinate Reference System

The input DEM should have an assigned CRS.

If the CRS is missing:

- The tool may be able to perform local raster operations.

- Distance-dependent or area-dependent calculations may not be reliable.

- The user should assign or define the correct CRS before further hydrological processing.

If the DEM uses a geographic CRS, latitude/longitude units are in degrees. This may be acceptable for some local, cell-based operations, but it is unsuitable for many calculations involving:

- Distances.

- Areas.

- Breach lengths.

- Cell-size measurements in metres.

- Resolution-based thresholds.

Where metric calculations are required, reproject the DEM to a suitable projected CRS and review the resampling method.

## 6.3 Resolution

The native raster resolution should be retained and documented. Mayim Tools uses resolution-aware parameters in selected tools, but resolution adaptation does not remove the need for professional review.

A 1 m LiDAR DEM and a 30 m global DEM should not automatically be interpreted as equivalent surfaces.

## 6.4 Vertical Accuracy

Where available, provide a formal vertical accuracy value such as:

- RMSE.

- LE90.

- A source-specific accuracy estimate.

- A survey-derived accuracy assessment.

If no formal value is available, the tool may use a conservative estimate based on the selected source type. The estimate must be treated as metadata guidance, not as a measured accuracy result.

# 7. Outputs, Reports and Provenance

## 7.1 Raster Outputs

Mayim Tools may produce:

- Processed DEMs.

- Classification masks.

- Artifact masks.

- Difference rasters.

- Smoothing masks.

- Diagnostic rasters.

Each raster output should be reviewed for:

- CRS.

- Extent.

- Resolution.

- NoData handling.

- Data type.

- Unexpected values.

- Alignment with the source DEM.

## 7.2 Text Reports

Text reports provide a human-readable summary of:

- Input data.

- Parameters.

- Warnings.

- Processing results.

- Output file locations.

- Recommended next steps.

- Interpretation guidance.

Text reports should be archived with the corresponding output rasters.

## 7.3 Provenance JSON

The provenance JSON is intended for machine-readable auditability. It may contain:

- Tool name.

- Algorithm ID.

- Tool version.

- Timestamp.

- Input path.

- Processing parameters.

- Output paths.

- Statistics.

- Warnings.

- Processing decisions.

The provenance file should be retained whenever the result is used in a formal engineering or analytical workflow.

## 7.4 Difference and Modification Outputs

A difference raster typically shows:

processed value - original value

Interpretation depends on the tool. For a smoothing operation:

- Negative values indicate a decrease.

- Positive values indicate an increase.

- Zero indicates no effective change.

Modification masks identify cells changed by the operation and support visual inspection and review.

# 8. Hydrology Tools

This chapter documents each Hydrology Tools algorithm currently available in Mayim Tools. Each tool follows the repeatable chapter structure described in Appendix A, covering purpose, pipeline stage, inputs, parameters, outputs, interpretation, and known limitations.

## 8.1 DEM Hydrological Screening

### Purpose

DEM Hydrological Screening implements the initial DEM review stages:

- Stage 0: Ingestion and QA.

- Stage 1: Artifact screening.

It is intended to be run before later smoothing, depression processing, flow-direction calculation, or flow-accumulation analysis.

Status:

Implemented in plugin version 0.2.0. Technical-preview implementation.

### Processing ID

mayimtools:demhydrologicalscreening

### Main Functions

The tool:

- Reads DEM metadata.

- Checks the assigned CRS.

- Reports raster dimensions.

- Reports native resolution.

- Reports NoData information.

- Assigns or estimates vertical accuracy.

- Detects NoData regions.

- Classifies void regions by size.

- Interpolates small voids where possible.

- Reports medium and large voids.

- Applies a local MAD-based artifact screening process.

- Produces screening masks.

- Produces a screened DEM.

- Produces a text report.

- Produces a JSON provenance record.

### Important Limitation

The current tool performs artifact screening, but it does not constitute a complete implementation of every possible Stage 1 correction method.

In particular, the following may remain separate or future functions:

- Full LiDAR ground filtering.

- SMRF implementation.

- Progressive TIN densification.

- Machine-learning bias correction.

- Bare-earth substitution.

- Co-registered land-cover correction.

The current implementation performs Stage 1 statistical artifact screening using a local Median Absolute Deviation approach. It does not yet provide complete source-specific correction for every DEM type.

SMRF ground filtering, progressive TIN densification, machine-learning bias correction, bare-earth substitution and co-registered land-cover correction remain future enhancements.

### Recommended Workflow

- Load the DEM into QGIS.

- Confirm that the DEM source type is known.

- Run DEM Hydrological Screening.

- Review the CRS warning or confirmation.

- Review the void mask.

- Review the artifact mask.

- Read the text report.

- Archive the provenance JSON.

- Decide whether smoothing or further artifact correction is justified.

- Do not proceed blindly to depression processing.

### Outputs

Typical outputs include:

<dem>_screened.tif
<dem>_void_mask.tif
<dem>_artifact_mask.tif
<dem>_screening_report.txt
<dem>_provenance.json
<dem>_screened.manifest.json

### Interpretation

A low artifact percentage does not prove that the DEM is correct. It indicates that relatively few cells were flagged by the selected MAD parameters.

A large void region should not be silently treated as valid terrain. It should be investigated as a possible data-collection gap.

A geographic CRS warning should be considered before operations requiring metric distances or areas.

## 8.2 DEM Hydrological Smoothing

### Purpose

DEM Hydrological Smoothing implements Stage 2: Controlled Smoothing.

It is intended to reduce residual high-frequency noise after Stage 1 screening, while preserving stronger terrain breaks.

Status:

Implemented in plugin version 0.2.0. Technical-preview implementation.

### Processing ID

mayimtools:demhydrologicalsmoothing

### Method

The current implementation uses Perona-Malik-style anisotropic diffusion.

This is an edge-preserving approach that reduces diffusion across stronger elevation gradients. It is therefore preferable to applying an indiscriminate uniform low-pass filter where preservation of terrain breaks is important.

### When to Use It

Use this tool only when:

- Stage 1 screening indicates residual high-frequency noise.

- The artifact mask and report have been reviewed.

- Smoothing is justified by the DEM characteristics.

- The analyst is prepared to inspect the difference and modification outputs.

Do not use smoothing simply because the DEM contains natural terrain variation.

### Recommended Starting Parameters

| Parameter | Recommended starting value | Valid range |
| --- | --- | --- |
| Diffusion iterations | 5 | 1–100 |
| Diffusion strength | 0.20 | 0.01–0.25 |
| Edge threshold | 1.0 elevation unit | Greater than 0 |
| Resolution scaling | Enabled | True / False |

### Parameter Guidance

Diffusion iterations

Start with 5 iterations. Use approximately 3–5 iterations for light noise. Consider 5–10 only where persistent high-frequency noise remains.

Increasing iterations increases the cumulative amount of smoothing and may alter genuine terrain features.

Diffusion strength

Start with 0.20. The current implementation limits this parameter to 0.25 for stability.

Higher values produce stronger updates per iteration, but stronger is not necessarily better.

Edge threshold

The edge threshold is expressed in the elevation units of the input DEM.

A lower value preserves terrain edges more strongly and reduces smoothing across elevation breaks. A higher value allows more smoothing across larger gradients and may reduce the distinction between noise and genuine terrain.

Where a reliable RMSE is available, an initial sensitivity test around one to two times the RMSE may be useful. This is only a starting heuristic and is not an automatic accuracy correction.

### Outputs

Typical outputs include:

<dem>_smoothed.tif
<dem>_smoothing_difference.tif
<dem>_smoothing_mask.tif
<dem>_smoothing_report.txt
<dem>_smoothing_provenance.json
<dem>_smoothed.manifest.json

### Interpretation

Review the smoothed DEM, the difference raster, and the modification mask together. Confirm that changes are concentrated in areas of expected high-frequency noise and that genuine terrain breaks (e.g. banks, channels, structures) have been substantially preserved.

A smoothing result should not be accepted purely on the basis that it looks visually cleaner. The difference raster should be checked against the artifact mask produced during Stage 1 screening to confirm the changes are consistent with the flagged noise.

The current implementation applies anisotropic diffusion to all valid cells rather than restricting smoothing only to cells flagged by the Stage 1 artifact mask. Users must therefore review the difference raster and modification mask carefully.

## 8.3 DEM Depression Analysis

### Purpose

DEM Depression Analysis implements Stages 3 and 4 of the Mayim Tools hydrological workflow.

It identifies candidate depressions, determines their basic properties, constructs a native depression hierarchy and classifies each depression using an inspectable artifact-likelihood score.

The tool does not modify DEM elevations.

### Pipeline stages

Stage 3 - Depression delineation and hierarchy construction
Stage 4 - Depression classification

### Processing ID

mayimtools:demdepressionanalysis

### When to use the tool

Run this tool after:

- DEM Hydrological Screening.

- Optional DEM Hydrological Smoothing.

- Review of the preceding reports and masks.

The input should be the best available artifact-screened or smoothed DEM for the analysis purpose.

### Processing method

The tool uses native Mayim components for:

- Priority-Flood-style depression detection.
- Spill-elevation identification.
- Depression feature calculation.
- In-house hierarchy construction.
- Multi-criteria artifact-likelihood scoring.

The tool does not fill, breach or otherwise modify the input DEM.

### Inputs

| Parameter | Description |
| --- | --- |
| Input DEM | DEM to analyse |
| Input MayimManifest | Optional manifest from a preceding tool |
| Vertical accuracy | RMSE or conservative vertical-accuracy estimate |
| Classification threshold | Threshold for high artifact likelihood |
| Review margin | Margin around the threshold for analyst review |
| Output folder | Folder for generated outputs |
| Load depression IDs | Loads the depression ID raster |
| Load classification | Loads the classification raster |

### Recommended starting values

| Parameter | Recommended value |
| --- | --- |
| Vertical accuracy | Use the value in the input manifest where available |
| Classification threshold | 0.60 |
| Review margin | 0.15 |
| Output folder | Permanent project-controlled folder |
| Load diagnostic rasters | Enabled for initial review |

### Classification results

The classifier uses three possible classes:

ARTIFACT
REAL_FEATURE
REVIEW_REQUIRED

Interpretation:

ARTIFACT:
  Candidate for Stage 5 selective enforcement.

REAL_FEATURE:
  Preserved by default.

REVIEW_REQUIRED:
  Preserved pending explicit analyst review.

The classification is based on inspectable criteria including:

- Depression depth relative to vertical accuracy.

- Depression area.

- Shape or compactness.

- Infrastructure evidence, where supplied.

- Known-basin evidence, where supplied.

- DSM-bias evidence, where supplied.

### Outputs

<dem>_depression_ids.tif
<dem>_depression_classification.tif
<dem>_depression_inventory.json
<dem>_depression_hierarchy.json
<dem>_depression_report.txt
<dem>_depression_provenance.json
<dem>_depression_analysis.manifest.json

### Raster classification values

Depression ID raster:

  0    = non-depression cell
  1+   = depression identifier
  -1   = NoData

Classification raster:

  0    = non-depression cell
  1    = ARTIFACT
  2    = REAL_FEATURE
  3    = REVIEW_REQUIRED
  255  = NoData

### Quality-control checks

Review:

- Depression count.

- Pit and spill elevations.

- Depression depth and area.

- Hierarchy structure.

- Classification score.

- Classification explanation.

- Classification raster.

- Review-required depressions.

- NoData behaviour.

- Input/output CRS and resolution.

### Limitations

The current implementation remains a technical-preview implementation. Further validation is required for:

- Complex nested depressions.
- Explicit merge-event tracking.
- Boundary-connected depressions.
- Multiple spill routes.
- Highly irregular terrain.
- Very large DEMs.
- Floating-point elevation ties.

The tool must be treated as analysis-only until the classification and hierarchy results have been reviewed.

## 8.4 DEM Hydrological Filling

### Purpose

DEM Hydrological Filling implements Stage 5 selective flow enforcement.

It consumes depression analysis results and applies a classification-driven decision:

ARTIFACT:
  De-pit, breach or fill according to the configured decision sequence.

REAL_FEATURE:
  Preserve unchanged.

REVIEW_REQUIRED:
  Preserve unchanged and report for analyst review.

### Processing ID

mayimtools:demhydrologicalfilling

### When to use the tool

Run this tool only after:

- DEM Depression Analysis has completed.

- The depression classification has been reviewed.

- The depression inventory has been checked.

- Review-required cases have been identified.

- The analyst accepts the Stage 5 enforcement parameters.

### Processing sequence

1. Isolated artifact pit:
   Apply single-cell de-pitting.

2. Larger artifact depression:
   Attempt constrained least-cost breaching.

3. Breach failure:
   Apply confined spill-level filling as fallback.

4. Real feature:
   Preserve unchanged.

5. Review-required feature:
   Preserve unchanged and record for review.

### Inputs

| Parameter | Description |
| --- | --- |
| Input DEM | DEM corresponding to the depression analysis |
| Input MayimManifest | Optional manifest from a preceding tool |
| Depression inventory | Inventory JSON from DEM Depression Analysis |
| Maximum breach length | Maximum permitted breach path length |
| Maximum breach depth | Maximum permitted local excavation depth |
| Connectivity | 4- or 8-connected enforcement |
| Output folder | Folder for generated outputs |
| Load preserved DEM | Loads the depression-preserving DEM |
| Load ready DEM | Loads the hydrology-ready DEM |
| Load decision raster | Loads the enforcement decision raster |

### Recommended starting values

| Parameter | Recommended value |
| --- | --- |
| Maximum breach length | 250 cells |
| Maximum breach depth | 5.0 elevation units |
| Connectivity | 8 |
| Output folder | Permanent project-controlled folder |
| Load outputs | Enabled for review |

These are starting values only. They must be reviewed relative to DEM resolution, vertical accuracy, terrain, depression size and project purpose.

### Outputs

<dem>_preserved_topology.tif
<dem>_hydrology_ready.tif
<dem>_enforcement_decisions.tif
<dem>_enforcement_difference.tif
<dem>_enforcement_report.txt
<dem>_enforcement_provenance.json
<dem>_enforcement.manifest.json

### Decision raster values

0    = unchanged non-depression cell
1    = artifact de-pitted
2    = artifact breached
3    = artifact filled as fallback
4    = real feature preserved
5    = review-required feature preserved
255  = NoData

The decision raster records the decision assigned to a depression footprint. It does not necessarily mean that every cell in that footprint changed elevation.

### Difference raster

The standard convention is:

hydrology-ready DEM - input DEM

Negative values = terrain lowered, generally by breaching
Positive values = terrain raised, generally by filling or de-pitting
Zero            = no elevation change

### Quality-control checks

Review:

- The preserved-topology DEM.

- The hydrology-ready DEM.

- The decision raster.

- The difference raster.

- Per-depression audit records.

- Breach paths.

- Filled areas.

- Review-required depressions.

- Total elevation change.

- NoData behaviour.

### Limitations

The current implementation is a technical-preview implementation.

The current preserved and hydrology-ready outputs may be identical for some decision sets because real and review-required depressions are conservatively preserved. Do not describe the hydrology-ready output as fully drained unless the report confirms that all relevant depressions were resolved.

The confined fill implementation is currently a confined spill-level operation. It should not be interpreted as a dynamic fill-spill simulation.

## 8.5 DEM Gradient Resolution

### Purpose

DEM Gradient Resolution implements Stage 6 flat-area resolution.

It identifies equal-elevation flat regions and applies a small synthetic gradient so that downstream flow-direction algorithms can assign a consistent direction across those regions.

The input DEM is never overwritten.

### Processing ID

mayimtools:demgradientresolution

### When to use the tool

Run this tool after:

- DEM Hydrological Screening.
- Optional DEM Hydrological Smoothing.
- DEM Hydrological Filling, if filling has introduced flats.

It should be run before flow-direction and flow-accumulation processing where unresolved flats would prevent reliable routing.

### Processing method

The tool uses native Mayim components based on the Garbrecht-Martz dual-gradient approach. The workflow is:

- Identify equal-elevation flat candidate cells.

- Identify connected flat regions.

- Identify higher-boundary cells.

- Identify lower-boundary cells.

- Calculate region-specific distance gradients.

- Combine the gradients to create a small synthetic surface gradient.

- Limit the correction using the supplied cell size and vertical accuracy.

- Preserve non-flat and NoData cells.

- Record unresolved flat regions for analyst review.

The tool does not fill or breach depressions. It only resolves flat areas that may prevent downstream flow-direction processing.

### Recommended settings

| Parameter | Recommended value | Notes |
| --- | --- | --- |
| Vertical accuracy | Use the value from the input manifest | Use a documented RMSE, LE90 or conservative estimate |
| Cell size | Native DEM resolution | Use the actual raster cell size |
| Connectivity | 8 | Use 4 only where the project methodology requires it |
| Load diagnostic outputs | Enabled | Recommended for initial review |
| Output folder | Permanent project-controlled folder | Avoid relying on temporary outputs |

### Outputs

<dem>_gradient_resolved.tif
<dem>_flat_mask.tif
<dem>_flat_region_ids.tif
<dem>_gradient_difference.tif
<dem>_gradient_report.txt
<dem>_gradient_provenance.json
<dem>_gradient.manifest.json

### Output interpretation

Gradient-resolved DEM

The gradient-resolved DEM is the Stage 6 output surface. A small synthetic gradient is introduced across resolved flat regions. The input DEM is not overwritten.

The output should be reviewed before it is used for:

- Flow-direction calculation.

- Flow-accumulation calculation.

- Stream-network extraction.

- Watershed delineation.

- Other terrain-based hydrological modelling.

Flat mask

0    = not identified as a flat candidate
1    = flat candidate cell
255  = NoData

The flat mask identifies cells considered for Stage 6 resolution. It does not indicate that every cell was necessarily modified.

Flat-region ID raster

0    = not part of a flat region
1+   = connected flat-region identifier
-1   = NoData

Each positive integer identifies a separate connected flat region. The region ID raster is useful for investigating whether disconnected flat areas were processed independently.

Gradient difference raster

The difference raster is calculated as:

gradient-resolved DEM - input DEM

Negative values = elevation lowered
Positive values = elevation raised
Zero            = no effective change

The difference raster should be inspected for:

- Unexpectedly large corrections.

- Corrections outside identified flat regions.

- Corrections near NoData boundaries.

- Changes that affect known banks, channels, roads or structures.

- Patterns indicating that a flat region was incorrectly identified.

Text report

The text report contains:

- Input DEM information.

- Cell size.

- Vertical accuracy.

- Connectivity.

- Number of flat candidate cells.

- Number of connected flat regions.

- Higher- and lower-boundary counts.

- Number of modified cells.

- Maximum elevation correction.

- Unresolved flat regions.

- Output file locations.

- Method and IP statement.

- Recommended next steps.

JSON provenance

The provenance file records the processing details required for review and reproduction, including:

- Tool name.

- Algorithm ID.

- Tool version.

- Pipeline stage.

- Input DEM.

- Input manifest, where supplied.

- Processing parameters.

- Derived parameters.

- Flat-region statistics.

- Output paths.

- Warnings.

- Unresolved regions.

MayimManifest

The manifest provides the standard interchange record for the next tool in the pipeline. It records the output raster and relevant inherited metadata, including:

- CRS.

- Cell size.

- Vertical accuracy.

- NoData value.

- Raster dimensions.

- Data type.

- Producing tool.

- Parent provenance identifier.

- Audit-log path.

### Quality-control checks

After running DEM Gradient Resolution:

- Confirm that the input DEM was not overwritten.

- Confirm that the output CRS matches the input CRS.

- Confirm that the output extent matches the input extent.

- Confirm that the output resolution matches the input resolution.

- Review the flat mask.

- Review the flat-region ID raster.

- Review the gradient difference raster.

- Confirm that NoData areas remain NoData.

- Confirm that non-flat cells remain unchanged.

- Confirm that the maximum correction is within the documented limit.

- Review unresolved flat regions in the report.

- Check the provenance JSON and manifest.

- Compare the resolved DEM with known terrain features.

- Confirm that the result is appropriate for the intended flow-routing method.

### Unresolved flat regions

A flat region may remain unresolved when no valid lower boundary is detected.

This does not necessarily indicate a software failure. Possible causes include:

- The outlet lies outside the DEM extent.

- A NoData region blocks the apparent drainage route.

- The DEM contains an actual closed or isolated flat area.

- The flat-region candidate is incorrectly segmented.

- The data contain equal or near-equal elevations that require further review.

- The chosen connectivity does not represent the intended drainage behaviour.

Unresolved regions are preserved unchanged and recorded in:

<dem>_gradient_report.txt
<dem>_gradient_provenance.json
<dem>_gradient.manifest.json

Users should not describe the DEM as completely gradient-resolved while unresolved regions remain.

### Limitations

The current technical-preview implementation requires further validation for:

- Multiple-outlet flat regions.

- Flats adjoining NoData.

- Flats touching the raster boundary.

- Rectangular raster cells.

- Floating-point near-equality tolerance.

- Very large flat regions.

- Flats introduced by complex Stage 5 filling.

- Directional correctness of the imposed gradient.

- Compatibility with every downstream flow-routing implementation.

The current implementation uses a region-aware workflow, but it should still be validated using representative test DEMs and project-specific terrain.

### Method and IP statement

DEM Gradient Resolution is an original Mayim implementation based on the Garbrecht-Martz flat-resolution methodology and the Mayim hydrological conditioning framework.

The tool does not call WhiteboxTools, RichDEM, TauDEM or another third-party hydrological implementation at runtime. Generic support libraries may be used for raster I/O and numerical operations, but the Stage 6 decision and terrain-modification logic remains within Mayim Tools.

# 9. Recommended Hydrological Workflow

The current Mayim Tools workflow is organised as a sequence of independent tools. Each stage should be reviewed before the next stage is run.

### Stage 0 and Stage 1 — DEM Hydrological Screening

- Load the source DEM into QGIS.

- Confirm the DEM source type.

- Run DEM Hydrological Screening.

- Review the CRS and resolution.

- Review the void classification mask.

- Review the artifact mask.

- Read the screening report.

- Investigate large void regions.

- Retain the screening provenance and manifest.

### Stage 2 — DEM Hydrological Smoothing

Run DEM Hydrological Smoothing only when the Stage 1 review indicates that residual high-frequency noise is present.

- Use the screened DEM or another reviewed DEM.

- Use the MayimManifest where available.

- Begin with the recommended smoothing parameters.

- Review the smoothed DEM.

- Review the smoothing difference raster.

- Review the smoothing mask.

- Confirm that genuine terrain breaks remain preserved.

- Retain the smoothing report, provenance and manifest.

### Stages 3 and 4 — DEM Depression Analysis

Run DEM Depression Analysis after the Stage 0–2 outputs have been reviewed.

- Supply the reviewed DEM.

- Supply the preceding MayimManifest where available.

- Detect candidate depressions.

- Review depression IDs.

- Review pit and spill information.

- Review depression features.

- Review the hierarchy output.

- Review the classification raster.

- Identify REVIEW_REQUIRED depressions.

- Retain the inventory, hierarchy, report, provenance and manifest.

The tool does not modify elevation values.

### Stage 5 — DEM Hydrological Filling

Run DEM Hydrological Filling only after the Stage 3/4 classification has been reviewed.

- Supply the same DEM used for depression analysis.

- Supply the depression inventory.

- Confirm that the matching depression ID raster is available.

- Review the breach-length and breach-depth limits.

- Confirm the connectivity setting.

- Run the tool.

- Review the decision raster.

- Review breach and filling changes.

- Review preserved real features.

- Review unresolved cases.

- Retain both DEM surfaces and their audit records.

The tool applies the following decision sequence:

ARTIFACT:
  de-pit if isolated
  otherwise attempt constrained breaching
  otherwise use confined filling fallback

REAL_FEATURE:
  preserve unchanged

REVIEW_REQUIRED:
  preserve unchanged and report for analyst review

### Stage 6 — DEM Gradient Resolution

Run DEM Gradient Resolution after Stage 5 where flat surfaces remain or have been introduced by filling.

- Supply the appropriate Stage 5 DEM.

- Supply the Stage 5 MayimManifest where available.

- Confirm vertical accuracy and cell size.

- Detect flat regions.

- Review the flat-region ID raster.

- Resolve suitable flat regions.

- Review the gradient difference raster.

- Investigate unresolved regions.

- Retain the report, provenance and manifest.

### Stage 7 — Hydrography Enforcement

Stage 7 is planned as an optional operation.

Where trusted vector hydrography is available:

- Review its source, date, CRS and positional accuracy.

- Compare it with the DEM-derived drainage.

- Identify material disagreements.

- Enable hydrography enforcement only where justified.

- Review adaptive burn-depth outputs.

- Review conflicts and unresolved areas.

- Retain the hydrography report and provenance.

### Stage 8 — Validation and Audit

A future pipeline-level audit tool will combine the individual stage outputs and produce a consolidated audit package.

Until that tool is available, retain:

- All stage-level reports.
- All provenance JSON files.
- All MayimManifest files.
- All difference rasters.
- All modification or decision masks.
- All unresolved-review records.
- The exact parameters used for each stage.

# 10. Quality Assurance and Interpretation

Every Mayim Tools output should be treated as an input to professional review, not as a final engineering result by itself.

### General QA discipline

For every tool:

- Confirm the input and output CRS.

- Confirm the input and output resolution.

- Confirm the extent and alignment.

- Confirm NoData handling.

- Review the Processing feedback.

- Review all warnings.

- Review the text report.

- Review the provenance JSON.

- Retain the MayimManifest.

- Compare differences with known terrain features.

- Record any parameter changes from the recommended values.

- Record the engineering reason for accepting or rejecting the output.

### Stage 0 and Stage 1 QA

Review:

- DEM source type.

- Vertical-accuracy assumption.

- Void counts.

- Void-region sizes.

- Interpolated small voids.

- Medium and large voids.

- Artifact-mask distribution.

- Potential false positives around terrain breaks.

### Stage 2 QA

Review:

- Diffusion iterations.

- Diffusion strength.

- Edge threshold.

- Resolution-scaling behaviour.

- Spatial distribution of changes.

- Maximum and mean elevation changes.

- Changes near channels, banks, roads and structures.

### Stages 3 and 4 QA

Review:

- Depression count.

- Pit elevations.

- Spill elevations.

- Depression depths.

- Areas and volumes.

- Boundary-connected cases.

- Nested relationships.

- Classification scores.

- Score contributions.

- REVIEW_REQUIRED cases.

### Stage 5 QA

Review:

- Decision-code raster.

- De-pitted cells.

- Breach paths.

- Breach lengths.

- Excavation depths.

- Filled cells.

- Preserved real features.

- Review-required features.

- Total elevation changes.

- Differences between preserved and hydrology-ready surfaces.

### Stage 6 QA

Review:

- Flat candidate count.

- Connected-region count.

- Higher and lower boundaries.

- Region-specific correction magnitudes.

- Unresolved regions.

- Difference raster.

- NoData boundaries.

- Whether terrain edges remain credible.

### Professional interpretation

A successful tool execution means that the software completed its requested operation. It does not establish that:

- The source DEM is accurate.

- The selected source type is correct.

- The vertical-accuracy estimate is correct.

- The classification is physically correct.

- A terrain modification is appropriate for the project.

- The result is suitable for design or regulatory use.

Independent engineering judgement remains required.

# 11. Troubleshooting

### Tool does not appear in the Processing Toolbox

Check:

- The plugin is enabled.

- The Mayim Tools provider is present.

- The tool is registered in the category file.

- QGIS was restarted after deployment.

- The QGIS log contains no import error.

- The deployed file exists in the QGIS 4 plugin directory.

### Tool appears but will not open

Check:

- The tool module imports correctly.

- All required native modules exist.

- The Processing algorithm ID is unique.

- The QGIS log contains no traceback.

- The plugin was deployed from the current source directory.

### Outputs are not created

Check:

- The output folder exists.

- The output folder is writable.

- The input DEM is valid.

- The input manifest path is correct.

- Required inventory or raster files exist.

- The Processing feedback contains a detailed error.

### Outputs are created but not loaded

Check:

- The relevant load checkbox is enabled.

- The output file exists and has non-zero size.

- The raster is valid in QGIS.

- The output CRS and data type are supported.

- The QGIS log contains no layer-loading warning.

### Report link does not open

Check:

- The report file exists.

- The displayed file URI is correct.

- The output folder was not temporary and subsequently cleaned.

- The report path is accessible in Windows Explorer.

### Stage 6 reports unresolved flat regions

This may be expected. Review:

- The flat-region ID raster.

- The DEM boundary.

- NoData areas.

- Lower-boundary detection.

- Connectivity.

- The original terrain.

- Whether the region is a genuine closed feature.

Do not automatically fill unresolved flats without an explicit and documented engineering decision.

### QGIS tests fail with No module named qgis

Tests that import QGIS classes must be run inside a QGIS-aware test environment. Native NumPy and standard-library tests can be run through the configured standalone test workflow with automatic pytest plugin loading disabled.

Do not install QGIS Python bindings using pip.

# 12. Versioning and Change Management

Mayim Tools and this manual are versioned independently but should be reviewed together where practical.

Current status:

Plugin version: 0.2.0
Manual version: 0.2.0
Status: Active development / Technical Preview

The plugin follows:

MAJOR.MINOR.PATCH

### MAJOR

Breaking changes, such as:

- Changing an algorithm ID.

- Removing an output.

- Changing the meaning of an output.

- Removing a required parameter.

- Breaking existing models or scripts.

### MINOR

Backwards-compatible features, such as:

- Adding a new tool.

- Adding a new category.

- Adding an optional parameter.

- Adding an optional computational backend.

### PATCH

Bug fixes and small improvements, such as:

- Correcting NoData handling.

- Correcting a report label.

- Improving error handling.

- Correcting documentation.

- Improving performance without changing the method.

New tools are added as numbered subsections under Section 8 and must follow the repeatable tool-chapter structure.

Each new tool chapter should document:

- Purpose.
- Pipeline stage.
- Processing method.
- Inputs.
- Recommended parameters.
- Outputs.
- Output interpretation.
- Quality-control checks.
- Limitations.
- Provenance.
- Related tools.

# 13. References

- QGIS Processing Framework documentation.

- PyQGIS API documentation for QGIS 4.

- GDAL raster data model documentation.

- Barnes, Lehman and Mulla (2014), Priority-Flood methodology.

- Barnes (2016), parallel Priority-Flood methodology.

- Barnes, Callaghan and Wickert (2020), depression hierarchy methodology.

- Garbrecht and Martz (1997), flat-area drainage-direction methodology.

- Lindsay and Creed (2005, 2006), minimum-impact and artifact-depression methodology.

- Lindsay and Dhun (2015), LiDAR drainage-pattern modelling.

- Lindsay (2016), hybrid breaching and sink-removal methodology.

- Perona and Malik (1990), anisotropic diffusion methodology.

- Pingel, Clarke and McBride (2013), SMRF methodology.

- Soille, Vogt and Colombo (2003), adaptive drainage enforcement.

- Wang and Liu (2006), depression identification and filling methodology.

- Mayim Tools DEM Hydrological Conditioning Research Paper, Revision 1.

# Appendix A — Processing Parameter Conventions

Where practical, Mayim Tools parameters follow these conventions:

- Distances and lengths are expressed in the linear unit of the input CRS (typically metres for a projected CRS).

- Elevation-based thresholds are expressed in the vertical unit of the input DEM.

- Boolean parameters default to the safer or more conservative setting where a default is required.

- Iteration counts and strength parameters include a stated valid range and a recommended starting value.

Repeatable Chapter Structure

Each future tool chapter under Section 8 (Hydrology Tools) will use the following structure:

- 1. Purpose

- 2. Pipeline stage

- 3. When to use the tool

- 4. Processing method

- 5. Required inputs

- 6. Optional inputs

- 7. Recommended parameter values

- 8. Processing steps

- 9. Outputs

- 10. Output interpretation

- 11. Quality-control checks

- 12. Warnings and limitations

- 13. Example workflow

- 14. Provenance and reporting

- 15. Related tools and next steps

This will keep the manual consistent as more tools are added.

# Appendix B — Output Naming Conventions

Output files generally follow the pattern <dem>_<suffix>.<ext>, where <dem> is the base name of the input DEM. Current suffixes in use:

| Suffix | Produced by | Description |
| --- | --- | --- |
| _screened.tif | DEM Hydrological Screening | Screened DEM raster |
| _void_mask.tif | DEM Hydrological Screening | NoData void classification mask |
| _artifact_mask.tif | DEM Hydrological Screening | MAD-based artifact mask |
| _screening_report.txt | DEM Hydrological Screening | Human-readable screening report |
| _provenance.json | DEM Hydrological Screening | Machine-readable provenance record |
| _smoothed.tif | DEM Hydrological Smoothing | Smoothed DEM raster |
| _smoothing_difference.tif | DEM Hydrological Smoothing | Processed minus original elevation difference |
| _smoothing_mask.tif | DEM Hydrological Smoothing | Modified-cell mask |
| _smoothing_report.txt | DEM Hydrological Smoothing | Human-readable smoothing report |
| _smoothing_provenance.json | DEM Hydrological Smoothing | Machine-readable provenance record |
| _depression_ids.tif | DEM Depression Analysis | Depression identifier raster |
| _depression_classification.tif | DEM Depression Analysis | Depression classification raster |
| _depression_inventory.json | DEM Depression Analysis | Depression inventory record |
| _depression_hierarchy.json | DEM Depression Analysis | Depression hierarchy record |
| _depression_report.txt | DEM Depression Analysis | Human-readable depression analysis report |
| _depression_provenance.json | DEM Depression Analysis | Machine-readable provenance record |
| _depression_analysis.manifest.json | DEM Depression Analysis | MayimManifest interchange record |
| _preserved_topology.tif | DEM Hydrological Filling | Depression-preserving DEM |
| _hydrology_ready.tif | DEM Hydrological Filling | Hydrology-ready DEM |
| _enforcement_decisions.tif | DEM Hydrological Filling | Enforcement decision raster |
| _enforcement_difference.tif | DEM Hydrological Filling | Processed minus original elevation difference |
| _enforcement_report.txt | DEM Hydrological Filling | Human-readable filling report |
| _enforcement_provenance.json | DEM Hydrological Filling | Machine-readable provenance record |
| _enforcement.manifest.json | DEM Hydrological Filling | MayimManifest interchange record |
| _gradient_resolved.tif | DEM Gradient Resolution | Gradient-resolved DEM |
| _flat_mask.tif | DEM Gradient Resolution | Flat candidate mask |
| _flat_region_ids.tif | DEM Gradient Resolution | Connected flat-region identifier raster |
| _gradient_difference.tif | DEM Gradient Resolution | Processed minus original elevation difference |
| _gradient_report.txt | DEM Gradient Resolution | Human-readable gradient resolution report |
| _gradient_provenance.json | DEM Gradient Resolution | Machine-readable provenance record |
| _gradient.manifest.json | DEM Gradient Resolution | MayimManifest interchange record |

# Appendix C — Glossary

| Term | Definition |
| --- | --- |
| Artifact correction | Improving a DEM so that it better represents the underlying terrain. |
| Hydrological enforcement | Modifying terrain so that flow-routing algorithms can operate consistently. |
| MAD | Median Absolute Deviation; a robust statistic used for local artifact screening. |
| NoData void | A region of a raster with no valid elevation value. |
| Provenance record | A machine-readable log of the inputs, parameters, and decisions applied during a processing run. |
| Anisotropic diffusion | An edge-preserving smoothing technique (Perona-Malik) that limits diffusion across strong gradients. |
| Difference raster | A raster representing processed value minus original value, used to visualise the effect of a tool. |
| RMSE | Root Mean Square Error; a common measure of vertical accuracy for elevation data. |
| LE90 | Linear Error at the 90th percentile; a vertical accuracy measure. |
| CRS | Coordinate Reference System. |
| Artifact | An elevation value or pattern that does not adequately represent the underlying terrain. Examples include vegetation remnants, building remnants, radar noise, striping and interpolation errors. |
| Depression | A cell or group of cells from which modelled surface water cannot escape because all available surrounding routes are higher. |
| Depression hierarchy | A representation of nested and connected depressions, including parent-child relationships, spill elevations and merge connectivity. |
| DEM | Digital Elevation Model. A raster dataset representing elevation. |
| DSM | Digital Surface Model. An elevation model that may include vegetation, buildings and other above-ground objects. |
| DTM | Digital Terrain Model. An elevation model intended to represent the bare-earth surface. |
| Flat region | A connected group of equal or near-equal elevation cells requiring additional gradient information before a unique flow direction can be assigned. |
| Hydrological conditioning | The preparation or modification of a DEM so that flow-routing operations produce hydrologically consistent results. |
| Manifest | A standard JSON interchange record containing raster metadata and provenance identifiers that link one Mayim Tools stage to the next. |
| Review-required | A classification or processing state indicating that available evidence is insufficient for automatic modification and requires analyst review. |
| Spill elevation | The lowest elevation at which water can leave a depression and enter an adjacent drainage area or higher-level depression. |
| Void | A NoData area in a raster dataset. |
| NoData | A raster value indicating that no valid observation exists for a cell. |
| Provenance | A structured record of the input data, parameters, methods, decisions, warnings and outputs associated with a processing operation. |

# Appendix D — Current Tool Summary

| Tool | Processing ID | Stages | Status |
| --- | --- | --- | --- |
| DEM Hydrological Screening | mayimtools:demhydrologicalscreening | 0–1 | Implemented |
| DEM Hydrological Smoothing | mayimtools:demhydrologicalsmoothing | 2 | Implemented |
| DEM Depression Analysis | mayimtools:demdepressionanalysis | 3–4 | Implemented technical preview |
| DEM Hydrological Filling | mayimtools:demhydrologicalfilling | 5 | Implemented technical preview |
| DEM Gradient Resolution | mayimtools:demgradientresolution | 6 | Implemented technical preview |
| DEM Hydrography Enforcement | Planned | 7 | Planned |
| DEM Pipeline Audit | Planned | 8 | Planned |

End of Mayim Tools User Manual

Plugin version: 0.2.0
Manual version: 0.2.0
Status: Active development / Technical Preview
Repository: https://github.com/chrismayim/mayim-tools
Licence: GNU General Public License v2 or later